DAILY ASSESSMENT FORMAT

| Date: | 27/05/2020 | Name: | Kirti B S |
| --- | --- | --- | --- |
| Course: | Signals & system | USN: | 4AL18EC026 |
| Topic: | 1. Boolean equations for digital circuits. Combinational circuits: Conversion of MUX and Decoders to logic gates. 2. Design of 7 segment decoder with common anode display | Semester & Section: | 3rd Sem ‘A’ Section |
| Github Repository: | Kirti BS |  |  |

FORENOON SESSION

Image of the session

REPORT

- Boolean equations for digital circuits.

- Boolean Algebra is the mathematical foundation of digital circuits.

- Boolean Algebra specifies the relationship between Boolean variables which is used to design combinational logic circuits using Logic Gates.

- The truth table shows a logic circuit's output response to all of the input combinations.

- A Boolean Variable takes the value of either 0 (False) or 1 (True).

- Symbols are used to represent Boolean variables e.g. A, B, C, X, Y, Z

- There are three basic logic operations AND, OR, NOT

- The Boolean Operators are • + ‾

    - A + B means A OR B

    - A • B means A AND B

    - A means NOT A

    - Nodes in a circuit are represented by Boolean Variables

- The most practical use of Boolean algebra is to simplify Boolean expressions which means less logic gates are used to implement the combinational logic circuit.

- Combinational circuits:

- Combinational Logic Circuits are memoryless digital logic circuits whose output at any instant in time depends only on the combination of its inputs

- The outputs of Combinational Logic Circuits are only determined by the logical function of their current input state, logic “0” or logic “1”, at any given instant in time.

- Combinational Logic Circuits are made up from basic logic NAND, NOR or NOT gates that are “combined” or connected together to produce more complicated switching circuits.

- These logic gates are the building blocks of combinational logic circuits.

- An example of a combinational circuit is a decoder, which converts the binary code data present at its input into a number of different output lines, one at a time producing an equivalent decimal code at its output.

- Conversion of MUX and Decoders to logic gates.

- A multiplexer is a device which allows one of a number of inputs to be routed to a single output.

- Multiplexers are useful in many situations. For example, in a CPU, data being written to memory might come from one of a number of sources - from a register, from the result of a calculation, etc - so a multiplexer would be used to select data from the appropriate source.

- Design of 7 segment decoder with common anode display.

AFTERNOON SESSION

Image of the session

REPORT

- Graphical user interfaces with Tkinter

- Introduction to Tkinter

- Setting up a GUI Widgets

- Connecting GUI widgets with callback functions

- Create multi-widget GUI

- Interacting with databases

- Introduction to “python with databases”

- Connecting and inserting data to SQLite via python

- Selecting, inserting, deleting and updating SQLite records

- Introduction to postdegreeSQL  psycopg2

- Selecting, inserting, deleting, and updating postdegreeSQL records

- Querying data from a MySQL databases